# C-410A Maintenance History Extract

Site: North Packaging Plant | Asset: C-410A rotary screw compressor | Extract date: 2026-05-13

# 1. Work Order Summary

| WO | date | asset | scope | reported findings | parts used | reading recorded | closeout note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WO-87941 | 2026-02-03 | C-410A | Quarterly PM | Coalescing filter DP trend rising but within watch band. | CF-410-COAL element not changed | separator DP 6.8 psid | Next change scheduled at major service. |
| WO-88412 | 2026-04-17 | M-410A | Motor inspection | Motor tag photo confirms 250 kW, FLA 392 A. | N/A | motor current 348 A at loaded condition | No bearing replacement performed. |
| WO-88990 | 2026-05-07 | C-410A | Service outage: filters and separator | Separator element carton label SEP-160-HF. Inlet filter IF-410-ELM installed. Oil filter OF-46 installed. Oil change recorded but quantity not entered in CMMS. | SEP-160-HF; IF-410-ELM; OF-46; PAO-46 | post-service separator DP 11.1 psid at local panel | Aftercooler wash deferred due to drain isolation issue. |
| WO-89004 | 2026-05-08 | C-410A | High temperature alarm response | Operator reported discharge temperature above panel alarm during high demand. Unit found in modulation mode. | no parts | discharge temperature 235 degF; separator DP 12.4 psid | Returned to load/unload after shift handover. |
| WO-89038 | 2026-05-09 | C-410A | Aftercooler inspection | Fin pack has lint mat on inlet face and warm discharge plenum. Wash kit staged; drain containment not ready. | cleaning kit staged | surface temperature difference 24 degF across cooler face | No cleaning completed. |
| WO-89120 | 2026-05-12 | C-410A | Vibration follow-up | Motor DE horizontal vibration repeat high. Coupling visual check completed; no guard rub found. | N/A | motor DE vibration 6.9 mm/s | Oil sample result pending at closeout. |

# 2. Technician Notes Needing Reconciliation

| topic | conflict or missing item | maintenance record statement |
| --- | --- | --- |
| separator element | Asset register still lists SEP-160-OLD. | WO-88990 records SEP-160-HF as installed on 2026-05-07. |
| control mode | Asset register records load/unload only. | WO-89004 says unit was found in modulation mode during high temperature alarm. |
| aftercooler status | Asset register says serviceable. | WO-89038 says wash kit staged but aftercooler wash not completed. |
| oil charge quantity | Service outage states oil changed. | No oil quantity was entered in CMMS for WO-88990. |
| motor rating | Motor current checks depend on confirmed rating. | WO-88412 confirms 250 kW and 392 A from motor tag photo. |

# 3. Service Reading Log

| date | asset_tag | measurement | component | value | unit | source note |
| --- | --- | --- | --- | --- | --- | --- |
| 2026-05-07 | C-410A | separator_dp | separator vessel | 11.1 | psid | post-service local panel |
| 2026-05-08 | C-410A | separator_dp | separator vessel | 12.4 | psid | high temperature alarm response |
| 2026-05-08 | C-410A | discharge_temperature | air-end discharge | 235 | degF | alarm response record |
| 2026-05-12 | C-410A | vibration_velocity_overall | motor DE bearing | 6.9 | mm/s | follow-up reading |
| 2026-05-09 | C-410A | aftercooler_face_delta | aftercooler | 24 | degF | surface check; not calibrated |
| 2026-05-07 | C-410A | oil_fill_quantity | compressor sump |  | L | quantity not entered |